Look at these evaluation datasets/benchmarks (AIS trajectory datasets, simulation benchmarks) and reinforcement learning / formal logic approaches.Excellent — the Imazu problem is exactly the kind of standard benchmark I was looking for. Let me get a couple more searches: formal logic/rule-encoding approaches, and LLM-based approaches, plus MPC-based approaches for completeness.Here's a structured map of the literature on COLREG-aware collision avoidance, organized by regulatory context → formalization approaches → algorithmic families → evaluation resources, with links so you can pull primary sources.

1. Regulatory backdrop (why this is a live research area)

IMO's Maritime Safety Committee completed a Regulatory Scoping Exercise (RSE) in 2021, assessing safety treaties including COLREG against four degrees of MASS autonomy (crewed-with-automation → remote-controlled-crewed → remote-controlled-uncrewed → fully autonomous), and identified provisions that apply to MASS without preventing operations, provisions that may need amendment or clarification, and provisions with gaps. Follow-on academic work concluded that COLREG could accommodate most MASS operations, but that Degree 3/4 (uncrewed) vessels likely need specific amendments.

This has now moved from scoping into an actual instrument: in May 2026, IMO's MSC adopted a new International Code of Safety for Maritime Autonomous Surface Ships (MASS Code), a goal-based framework requiring autonomous/remote-controlled ships to meet safety and environmental standards equivalent to conventional ships, alongside SOLAS compliance. It takes effect 1 July 2026 as a non-mandatory instrument, with a mandatory version targeted for adoption by July 2030. This is the regulatory context your agent will eventually need to sit inside — worth monitoring alongside the Q&A dataset.

2. Formalizing COLREG into machine-checkable rules

Since COLREG text is written for human judgment, several groups have tried to turn it into precise, executable specifications:

- Temporal logic formalization — Krasowski & Althoff formalized the two-power-driven-vessel collision prevention rules using temporal logic with modular predicates and adjustable parameters, validated against over 1,200 real vessels across three US coastal areas from AIS data. This is a good template if you want machine-checkable "ground truth" for your scenario-generation logic. (IEEE IV 2021)

- Deductive/theorem-proving verification — work using Isabelle/HOL toward formally verifying collision-avoidance control algorithms against COLREG properties (Foster, Gleirscher, Calinescu, ICECCS 2020).

- COLREGS compliance metrics — Woerner, Benjamin, Novitzky & Leonard (MIT) built a framework to quantify and evaluate the otherwise subjective nature of COLREGS, providing a path toward standardized evaluation and certification of protocol-constrained collision avoidance systems, and separately validated it with on-water field trials of up to five simultaneously interacting autonomous vessels across 10 complex scenarios (6,150+ vehicle-pair encounters), scored on spatial efficiency, temporal efficiency, protocol compliance, and safety. This MIT group's software stack is MOOS-IvP, a genuinely open-source, field-tested marine autonomy platform — worth a look if you want a real vehicle-in-the-loop testbed rather than a pure simulator.

- A useful engineering-side pushback worth reading: a GitHub issue thread arguing COLREGS as a rule system was written for human judgement, and encoded as data and evaluated by a pure function, it fails formal verification in classifiable ways — pairwise rules that don't compose for three vessels among them — i.e., the multi-vessel composability problem is real and unsolved, not just an implementation detail.

3. Algorithmic families for COLREG-compliant avoidance

Two recent survey/review papers are good entry points: a Frontiers in Robotics/AI survey on COLREGS coverage across approaches, and a 2026 Journal of Navigation review that organizes prior work into three families — classical search/optimisation, real-time reactive methods, and learning-based approaches — comparing them on rules compliance, computational cost, and onboard constraints.

a) Velocity Obstacle (VO) family — the most COLREG-native geometric approach, since VO naturally represents "the set of velocities that would lead to collision" and can be shaped to also encode give-way/stand-on obligations.

- Kuwata, Wolf, Zarzhitsky & Huntsberger's foundational VO+COLREGS paper (IEEE J. Oceanic Eng. 2014).

- Variants: NLVO, RVO, ORCA, GRVO adapted to ships, plus COLREG-specific extensions — e.g. an encounter-specific VO that classifies the vessel-to-vessel encounter under COLREGS Rules 13–15 and 17 first, then assigns an encounter-specific domain before formulating the VO, increasing robustness to non-compliant behaviour by the other vessel (Sci-Direct/NTNU Autoferry project).

- Recent hybrid: combining VO with Control Barrier Functions specifically for Rule 14 (head-on) and Rule 15 (crossing), showing improved computational efficiency over conventional VO-based methods (KAIST, 2025-26).

b) Model Predictive Control (MPC) — good fit for "early and substantial action" since it optimizes over a receding horizon with COLREG constraints baked into the cost/constraint function.

- Johansen, Cristofaro & Perez, scenario-based MPC (IFAC 2016).

- Eriksen, Breivik & Wilthil, "Model predictive control for autonomous ship collision avoidance: A systematic approach to COLREGS compliance" (IEEE Access 2019) — frequently cited as a rigorous, systematic COLREG-in-MPC treatment.

c) Reinforcement learning / Deep RL — currently the most active area by publication volume.

- Deep RL + LSTM in continuous action spaces, trained/tested against the Imazu problem (see §4) — Sawada, Sato & Majima, J. Marine Sci. Tech. 2021.

- PPO-based approaches combining reward shaping with CPA/ship-domain risk metrics (2023-2025, multiple groups, e.g. Frontiers 2025 PPO paper).

- Hierarchical RL combining high-level global intent planning with low-level fine rudder control, integrating multi-dimensional uncertainty modeling for dynamic and uncertain scenarios (2025).

- Multi-vessel/distributed approaches: a 2025 JMSE paper proposing a framework integrating quantitative COLREGs analysis with a distributed stochastic search mechanism for complex multi-ship environments, explicitly noting most prior work only handles simple two-ship encounters.

d) Classical search/optimization — A*, RRT (notably COLREGRRT, an RRT-based COLREGs-compliant motion planner, IEEE RA-L 2018), ant colony optimization, potential fields, Dynamic Window Approach — generally faster/more interpretable than RL but less adaptive to novel scenarios.

e) LLM-based / hybrid agentic approaches (closest to what you're building) — this is brand-new (2025-26):

- CORALL (github.com/Klins101/CORALL) — an open-source framework integrating LLMs (OpenAI GPT and Anthropic Claude models) with real-time DCPA/TCPA risk assessment for COLREGs-compliant collision avoidance in ASVs, validated across the 22 standardized Imazu test scenarios, with explainable reasoning and an LLM-vs-baseline comparison mode. Directly relevant as reference architecture.

- Nav-DLLC, a dual-LLM framework using ReAct-based prompting to decompose navigation tasks, with a LoRA-fine-tuned small LLM as the collision-avoidance core; reported 86% collision-avoidance accuracy and 90% behavior compliance, outperforming LLM baselines and traditional DWA/APF methods (2025).

- Navigation-GPT — a framework coupling ReAct prompting with LoRA, plus a proposed three-phase MASS deployment roadmap (core model integration, domain fine-tuning, integrated operations) (Journal of Navigation, 2026).

- Most directly useful to you: a benchmark dataset of 50 diverse real-world navigation scenarios built from AIS data, labeled with applicable COLREG rules, recommended actions, and reasoning for the action, used to evaluate multiple LLM architectures — finding the maritime navigation task remains difficult without fine-tuning, even for large frontier models (arXiv 2608.08281, 2026). This is essentially the "scenario reasoning" evaluation set you'd want — worth requesting/replicating.

4. Evaluation datasets & benchmarks

This is the area most people underinvest in, and where I'd point your attention:

| Resource | What it is | Why it matters |
| --- | --- | --- |
| Imazu problem | A standard set of 19 single-ship (non-close-quarters) encounter scenarios plus a harder 22-scenario benchmark spanning 1–4 target ships, originating from Japanese ship-handling research (Imazu et al.) | The closest thing the field has to a shared "test suite" — used across dozens of VO, MPC, and RL papers, so results are comparable. Start here for your agent's eval harness. |
| AIS-derived encounter-scenario extraction | Multiple 2023-2026 papers propose pipelines that mine real AIS trajectory data to auto-extract encounter scenarios, rank them by risk/exposure importance, and package them as benchmark cases for testing and evaluating collision avoidance systems of unmanned ships | Real-world statistical distribution of encounters, not hand-picked cases — good for stress-testing beyond the "textbook" Imazu scenarios. See the ScienceDirect "Ship encounter scenario generation for collision avoidance algorithm testing based on AIS data" (2023) and the 2026 "compressed trajectories" follow-up. |
| Generative AI scenario synthesis | A 2026 arXiv paper builds a generative framework for safety-critical encounter scenarios from a year of AIS records from a Singapore seaway, with performance criteria defined separately across tracking, collision-avoidance, and post-encounter-recovery trajectory segments | Points toward procedurally generating harder/rarer scenarios than what's naturally in AIS logs — relevant if you extend my templated scenario generator. |
| LLM-COLREG labeled scenario set | The 50-scenario AIS-derived, rule-labeled, reasoning-annotated dataset noted above (arXiv 2608.08281) | Directly the same shape as what you asked me to build, but grounded in real AIS tracks rather than hand-written exam questions — a natural complement to the 500-item Q&A set. |
| MIT "road test" framework | On-water field experimentation with up to five simultaneously interacting vessels, scored on spatial/temporal efficiency, protocol compliance and safety across 10 scenarios | The most rigorous physical evaluation methodology in the literature; useful as a target evaluation protocol if you ever move from sim to real/HIL testing. |
| CORALL repo's scenario harness | Open-source, runnable evaluation against the 22 Imazu scenarios with an LLM-vs-baseline comparison mode | Fastest path to an actual working eval loop for an LLM/agent-based collision avoidance system — worth cloning and adapting rather than building from scratch. |

5. Open challenges flagged repeatedly across this literature

- Multi-vessel composability — COLREG is written pairwise (give-way/stand-on between two vessels); nearly every survey and the GitHub formal-methods thread flag that extending to 3+ simultaneous vessels is unresolved and where most approaches diverge or get ad-hoc.

- "Fuzziness" of qualitative terms — surveys explicitly note COLREGs' fuzziness with regard to automation applications remains a challenge for MASS implementations even though IMO's RSE designated COLREGs as the reference standard for MASS collision avoidance — this is the same "safe speed," "ample time," "early and substantial" problem you flagged.

- Head-on angle threshold ambiguity — one paper notes explicitly that COLREGs do not define the precise angular threshold for a head-on encounter, so prior studies have used varying values such as ±15° or ±5° — a good concrete example of where you'll need to pick and document a design decision for your agent (my scenario dataset used a similar reasonable-default approach).

- Sim-to-real gap — most RL/VO papers validate only in simulation against Imazu-style scenarios; genuinely field-tested work (MIT's road test being the main exception) is rare.

- LLM reliability without fine-tuning — the 2608.08281 benchmark's headline finding, that frontier LLMs struggle with maritime navigation reasoning until fine-tuned, is directly relevant to your plan of training an agent on this domain rather than relying on prompting alone.

If useful, I can next: (a) pull and adapt the Imazu 22-scenario benchmark into the same JSON schema as your 500-item Q&A set so you have a unified eval harness, or (b) sketch the procedural (bearing × vessel-type × visibility → correct action) scenario generator I mentioned earlier, extended with the Rule 14 head-on threshold and multi-vessel composability caveats these papers raise.